// === Human/Object Following Car ===

// Ultrasonic + 2 IR sensors + L298N motor driver

#define IR_LEFT 2

#define IR_RIGHT 3

// === Ultrasonic Sensor Pins ===

#define TRIG 11

#define ECHO 12

// === Motor Driver Pins (L298N) ===

int enLeft  = 5;

int enRight = 6;

int left1 = 9;

int left2 = 10;

int right1 = 7;

int right2 = 8;

// === Settings ===

#define MOTOR_SPEED 150

#define DETECT_DISTANCE 2000   // 20 meters = 2000 cm

long duration;

int distance;

void setup() {

Serial.begin(9600);

Serial.println("Human/Object Following Car (Range: 20 meters)");

pinMode(IR_LEFT, INPUT);

pinMode(IR_RIGHT, INPUT);

pinMode(TRIG, OUTPUT);

pinMode(ECHO, INPUT);

pinMode(left1, OUTPUT);

pinMode(left2, OUTPUT);

pinMode(right1, OUTPUT);

pinMode(right2, OUTPUT);

pinMode(enLeft, OUTPUT);

pinMode(enRight, OUTPUT);

}

// === Measure distance using Ultrasonic sensor ===

int getDistance() {

digitalWrite(TRIG, LOW);

delayMicroseconds(2);

digitalWrite(TRIG, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG, LOW);

// Max timeout increased to handle large range (about 120 ms for 20 m)

duration = pulseIn(ECHO, HIGH, 120000);

distance = duration * 0.034 / 2;  // convert to cm

return distance;

}

// === Movement functions ===

void forward() {

digitalWrite(left1, HIGH);

digitalWrite(left2, LOW);

digitalWrite(right1, HIGH);

digitalWrite(right2, LOW);

analogWrite(enLeft, MOTOR_SPEED);

analogWrite(enRight, MOTOR_SPEED);

Serial.println("→ Moving Forward");

}

void left() {

digitalWrite(left1, HIGH);

digitalWrite(left2, LOW);

digitalWrite(right1, LOW);

digitalWrite(right2, HIGH);

analogWrite(enLeft, MOTOR_SPEED);

analogWrite(enRight, MOTOR_SPEED);

Serial.println("↰ Turning Left");

}

void right() {

digitalWrite(left1, LOW);

digitalWrite(left2, HIGH);

digitalWrite(right1, HIGH);

digitalWrite(right2, LOW);

analogWrite(enLeft, MOTOR_SPEED);

analogWrite(enRight, MOTOR_SPEED);

Serial.println("↱ Turning Right");

}

void stopCar() {

digitalWrite(left1, LOW);

digitalWrite(left2, LOW);

digitalWrite(right1, LOW);

digitalWrite(right2, LOW);

Serial.println("⛔ Stopped");

}

void loop() {

int leftState = digitalRead(IR_LEFT);

int rightState = digitalRead(IR_RIGHT);

int distance = getDistance();

Serial.print("Left: "); Serial.print(leftState);

Serial.print(" | Right: "); Serial.print(rightState);

Serial.print(" | Distance: "); Serial.print(distance);

Serial.println(" cm");

// Object detected within 20 meters (2000 cm)

if (distance <= DETECT_DISTANCE && distance > 0) {

if (leftState == LOW && rightState == LOW) {

forward();

}

else if (leftState == LOW && rightState == HIGH) {

left();

}

else if (rightState == LOW && leftState == HIGH) {

right();

}

else {

stopCar();

}

}

else {

stopCar(); // No object within 20 m

}

delay(100);

}